CAUTION

With the addition of DOSDISK there is one dangerous situation

that can occur when using TURBOSET.  If any drive is set to

MS-DOS format with DOSDISK, you CANNOT use TURBOSET until the

MS-DOS disk is freed back to CP/M.  At present TURBOSET does not

trap this situation and warn you what is happening.

So never use TURBOSET after DOSDISK without first running

DOSDISK  OFF

The reason for this is that the TurboRom only has one externally

settable DPB and both DOSDISK and TURBOSET use this parameter

block.  If Turboset sets the parameter block while DOSDISK is

still in memory unpredictable disk read/write problems will occur.

, saving one revolution per track.*//*4/17/84 DJM skeleton kaypro 10 logic*//*3/6/84 DJM new v